Status Report prepared under clause 7.12 of the Market Rules by System Management
1 January 2016 – 31 March 2016

Table of Contents

1	Introduction	3

1.1	System Management	3

1.2	Status Report	3

2	Issuance of Dispatch and Operating Instructions.	4

3	Non-compliance with Dispatch and Operating Instructions	5

4	Issuance of Dispatch Instructions to Balancing Facilities Out of Merit Order.	7

4.1	Out of Merit instances reported to the IMO	7

4.2	Other instances of Out of Merit dispatch	7

5	Transmission constraints	7

6	Operating States, Shortfalls in Ancillary Services and Involuntary Curtailment of Load	9

6.1	High Risk Operating State	9

6.2	Emergency Operating State	9

6.3	Shortfalls in Ancillary Services	14

6.4	Involuntary curtailment of load	14

7	LFAS Facilities out of Merit Order	15

- Introduction

## System Management

Western Power is established under section 4(1)(b) of the Electricity Corporations Act 2005 and has the functions conferred under section 41 of that act.

Part 9 of the Electricity Industry Act 2004 makes provision for a wholesale electricity market and provides for the establishment of Market Rules.

Regulation 13 of the Electricity Industry (Wholesale Electricity Market) Regulations 2004 provides that the Market Rules may confer on an entity the function of operating the SWIS in a secure and reliable manner.

Clause 2.2 of the Wholesale Electricity Market Amending Rules (September 2006) (Market Rules) confers this responsibility upon the segregated (“ring fenced”) business unit of Western Power known as System Management. Amongst these responsibilities, the functions of System Management are to:

- release information required by the Market Rules;

- monitor rule participants compliance with the Market Rules relating to dispatch and power system security and power system reliability; and

- provide regular reports to AEMO,  IMO and other market participants.

Included in the requirement to monitor and report is this Status Report, described in clause 7.12 of the Market Rules.

## Status Report

Clause 7.12 requires System Management to provide a quarterly report on the performance of the market with respect to the dispatch process. The report must include details of:

- the incidence and extent of issuance of Operating Instructions and Dispatch Instructions;

- the incidence and extent of non-compliance with Operating Instructions and Dispatch Instructions;

- the incidence and reasons for the issuance of Dispatch Instructions to Balancing Facilities Out of Merit, including for the purposes of this clause, issuing Dispatch Orders to the Verve Energy Balancing Portfolio in accordance with clause 7.6.2;

- the incidence and extent of transmission constraints;

- the incidence and extent of shortfalls in Ancillary Services, involuntary curtailment of load, High Risk Operating States and Emergency Operating States;

- the incidence and reasons for the selection and use of LFAS Facilities under clause 7B.3.8.

System Management has prepared this report pursuant to its obligations under clause 7.12 of the Market Rules, for the period 1 January 2016 to 31 March 2016.

Unless otherwise specified, data contained within this report refers to trading dates and not calendar dates.

- Issuance of Dispatch and Operating Instructions.

System Management issued a total of 10,902 Dispatch Instructions to Market Participants during the current reporting period.

Figure 1 below shows the number of Dispatch instructions issued by month since 
1 October 2015.

Figure 1: Dispatch Instructions per month

During the current reporting period, System Management issued a total of 37 Operating Instructions.

Under the Market Rules an Operating Instruction is required for Commissioning and Reserve Capacity Testing.

Figure 2 below shows the number of Operating Instructions issued by month since 1 October 2015.

Figure 2: Operating Instructions per month

- Non-compliance with Dispatch and Operating Instructions

In the current reporting period, System Management reported 414 instances of non-compliance with Dispatch Instructions by a Market Participant at the end of their scheduled interval taking into account the Tolerance Range where applicable.

System Management issued 15,088 one minute non-compliance notifications to Market Participants for non-compliance with Dispatch Instructions during the reporting period taking into account the Tolerance Range where applicable.

There were a total of 223 failures by a Market Participant to acknowledge a Dispatch Instruction through the Market Participant Interface.

There were no instances of failure by a Market Participant to acknowledge an Operating Instruction during the current reporting period.

Figure 3 below provides historical non-compliance data since 1 October 2015.

Figure 3: 	Dispatch Instruction non-compliance notifications (end of interval) and non-acknowledged Dispatch Instructions

- Issuance of Dispatch Instructions to Balancing Facilities Out of Merit Order.

## Out of Merit instances reported to the IMO

During the reporting period there were no self-reported occasions of potential non-compliant Out of Merit dispatch to the IMO that may have been in breach of the Market Rules.

## Other instances of Out of Merit dispatch

Section 5 of this report contains information pertaining to facilities that have been impacted by transmission constraints. Where a transmission constraint reported in section 5 has resulted in a facility/ies being dispatched to a position that differs from the applicable Balancing Merit Order then these instances will constitute Out of Merit dispatch. Dispatch Advisory notifications are released for these transmission constraint related instances.

Section 6 of this report describes occasions of High Risk and Emergency Operating States that have occurred during the reporting period. During elevated Operating States there may be a need to dispatch facilities Out of Merit to return the power system to a Normal Operating State where indicated in the information provided in section 6.

- Transmission constraints

A “transmission constraint” refers to the configuration of the transmission network that has an effect or potential effect of constraining or otherwise varying the output of a generator.  The resultant situation has a generation Facility either decrease output or increase output as required by the circumstances of the constraint.

System Management has identified the following transmission constraints during the period.

- On 7 January 2016 commencing Interval 7:2 until 11 January 2016 ending Interval 8:1 major bushfires threatened the 330kV and 132kV transmission lines in the South West Region. System Management replaced generation in the South West Region with generation in the Northern Region to reduce load on the fire affected assets. The following facilities were constrained on during the period (refer to Dispatch Advisories 15608, 15609, 15610, 15611, 15620, 15624, 15625, 15627, 15630, 15632, 15633);

  - NEWGEN_NEERABUP_GT1 constrained on for a total of 54 Intervals

  - PERTHENERGY_KWINANA_GT1 constrained on for a total of 24 Intervals

  - TIWEST_COG1 constrained on for 1 interval

- On 17 January 2016, commencing Interval 10:1 until 19 January 2016 ending Interval 6:1, lightning and storm activity threatened the MU_NGS_X1 and NGS_KDN_X1 220kV transmission lines. The NOR_MRT81 was taken out of service during the lightning and storm activity resulting in INVESTEC_COLLGAR_WF1 being constrained to 0MW for 48 Intervals during this period (Refer to Dispatch Advisories 15657, 15658 and  15661)

- On 23 January 2016, commencing Interval 16:2 until 24 January 2016 ending Interval 6:1 lightning and storm activity in the Eastern Goldfields region required the NOR_MRT81 to be taken out of service resulting in INVESTEC_COLLGAR_WF1 being constrained to 0MW for 8 Intervals (Refer to Dispatch Advisory 15787).

- On 23 January 2016, commencing Interval 19:2 and ending 24 January 2016 Interval 19:2 a pole failure resulted in the SNR_WGP_APJ81 line being out of service. This resulted in the ALCOA_WGP being constrained to 0MW for a total of 46 Intervals during the period. (Refer to Dispatch Advisory 15788)

- On 25 January 2016 commencing Interval 3:2 and ending Interval 17:2 severe lightning required the MRT_NOR81 line to be taken out of service which resulted in the INVESTEC_COLLGAR_WF1 and NAMKKN_MERR_SG1 being constrained to 0MW for 28 Intervals. (Refer to Dispatch Advisory 15826)

- On 28 January 2016 commencing Interval 16:2 and ending Interval 19:2 an overload on the APJ_PNJ_81 line required the facility ALINTA_PNJ_U1 to be constrained to a maximum of 120MW for a total of 7 Intervals.

- On 8 February 2016 commencing Interval 11:1 and ending 9 January 2016 Interval 6:1, due to major bushfires in the South West Region the LWT-SHO/KEM91 and KW-KEM/OLY91 330kV lines tripped. This was followed by a higher than forecasted load resulting in the need to Dispatch the following facilities to meet demand. (Refer to Dispatch Advisories 15867 and 15872)

  - ALCOA_WGP constrained on for 2 Intervals

  - ALINTA_PNJ_U1 constrained on for 2 Intervals

  - ALINTA_PNJ_U2 constrained on for 2 Intervals

  - ALINTA_WGP_GT constrained on for 5 intervals

  - BW1_BLUEWATERS_G2 constrained on for 2 Intervals

  - BW2_BLUEWATERS_G1 constrained on for 2 Intervals

  - MUJA_G3 constrained on for 4 Intervals

  - NEWGEN_NEERABUP_GT1 constrained on for 4 Intervals

  - TIWEST_COG1 constrained on for 2 Intervals

- On 24 March 2016 commencing Interval 14:1 and ending Interval 15:1 due to a scheduled outage at Merredin Terminal, INVESTEC_COLLGAR_WF1 was curtailed to a maximum of 70MW.

- Operating States, , Shortfalls in Ancillary Services and Involuntary Curtailment of Load

## High Risk Operating State

14 instances of a High Risk Operating States occurred during the report period.

| Date and Interval/s | 7 January 2016 Interval 7:2 to 11 January 2016 Interval 8:1 |
| --- | --- |
| DA Number | 15608 |
| Details | A major bushfire threatened the 330kV and 132kV transmission lines in the South West Region. The following lines were affected (MU-NT91; SHO-ST/OLY91; LWT-WGP91; LWT-WGP92; KW-KEM/OLY91; LWT-SHO/KEM91; LWT-ST91; PIC-PNJ/BSN/KEM81; SNR-WGP/APJ81; WGP-WOR81). |
| System Management action | Replaced generation in the South West Region with generation in the Northern Region to reduce the load on the fire affected assets. The facilities NEWGEN_NEERABUP_GT1, PERTHENERGY_KWINANA_GT1 and TIWEST_COG1 were constrained on throughout the period. |

| Date and Interval/s | 6 January 2016 Interval 23:1 |
| --- | --- |
| DA Number | 15606 |
| Details | NEWGEN_KWINANA_CCG1 tripped at 23:07 from exporting approximately 185MW and a frequency deviation to approximately 49.6Hz occurred. |
| System Management action | Continued to dispatch according to the latest BMO |

| Date and Interval/s | 6 January 2016 Interval 20:2 to 7 January 2016 Interval 6:1 |
| --- | --- |
| DA Number | 15603 |
| Details | Lightning and storm activity resulted in the loss of the MU-NGS-KDN line at approximately 20:42. Further complications would have arisen if the Non-Scheduled Generation in the Goldfields region fell below forecasted quantities. This may have required some Out of Merit Dispatch from WEST_KALGOORLIE_GT2, WEST_KALGOORLIE_GT3, STHRNCRS_EG and PRK_AG to maintain Power System Security and Reliability. |
| System Management action | No Out of Merit Generation was required and System Management continued to Dispatch as per the latest Balancing Merit Order |

| Date and Interval/s | 13 January 2016 Interval 16:2 to 14 January 2016 Interval 6:1 |
| --- | --- |
| DA Number | 15648 |
| Details | A major bushfire in the South West region caused a trip on the KW-KEM/OLY91 and LWT-SHO/KEM91 330kV transmission lines at approximately 16:58. |
| System Management action | No Out of Merit Generation was required and System Management continued to Dispatch as per the latest Balancing Merit Order |

| Date and Interval/s | 16 January 2016 Interval 13:2 to 16:2 |
| --- | --- |
| DA Number | 15654 |
| Details | A bushfire threatened the CT-MSS-PNJ81 transmission line in the South West Region. There was a possibility that generation would be constrained in the area to maintain Power System Security and Power System Reliability. |
| System Management action | No Out of Merit Generation was required and System Management continued to Dispatch as per the latest Balancing Merit Order |

| Date and Interval/s | 16 January Interval 16:2 to Interval 17:1 |
| --- | --- |
| DA Number | 15655 |
| Details | Lightning and storm activity threatened the MU-NGS-X1 and NGS-KDN-X1 220kV transmission lines. In the event of the loss of these transmission lines, there would be a requirement to place the Eastern Goldfields region on a radial and potentially constrain Non Scheduled Generation in the region. |
| System Management action | No Out of Merit Generation was required and System Management continued to Dispatch as per the latest Balancing Merit Order |

| Date and Interval/s | 17 January 2016 Interval 10:1 to Interval 13:2 |
| --- | --- |
| DA Number | 15657 |
| Details | Lightning and storm activity threatened the MU-NGS-X1 and NGS-KDN-X1 220kV transmission lines. In the event of the loss of these transmission lines, there would be a requirement to place the Eastern Goldfields region on a radial. The NOR-MRT81 line was taken out of service during the lightning and storm activity. |
| System Management action | INVESTEC_COLLGAR_WF1 was constrained to zero MW in order to maintain Power System Security and Power System Reliability |

| Date and Interval/s | 17 January 2016 Interval 14:1 to 18 January 2016 Interval 7:1 |
| --- | --- |
| DA Number | 15658 |
| Details | Lightning and storm activity threatened the MU-NGS-X1 and NGS-KDN-X1 220kV transmission lines. The loss of these transmission lines would require the Eastern Goldfields region to be placed on a radial. The NOR-MRT81 line was taken out of service during the lightning and storm activity. |
| System Management action | INVESTEC_COLLGAR_WF1 was constrained to zero MW in order to maintain Power System Security and Power System Reliability |

| Date and Interval/s | 18 January 2016 Interval 18:1 to 19 January 2016 Interval 6:1 |
| --- | --- |
| DA Number | 15661 |
| Details | Lightning and storm activity threatened the MU-NGS-X1 and NGS-KDN-X1 220kV transmission lines. In the event of the loss of these transmission lines, there would be a requirement to place the Eastern Goldfields region on a radial. The NOR-MRT81 line was taken out of service during the lightning and storm activity. |
| System Management action | INVESTEC_COLLGAR_WF1 was constrained to zero MW in order to maintain Power System Security and Power System Reliability |

| Date and Interval/s | 19 January 2016 Interval 17:2 to Interval 20:1 |
| --- | --- |
| DA Number | 15664 |
| Details | Due to a Real Time Dispatch Engine (RTDE) degradation, the RTDE was unable to send Dispatch Instructions to facilities. |
| System Management action | System Management issued verbal Dispatch Instructions to facilities based on the last received Balancing Merit Order (for interval 18:1) |

| Date and Interval/s | 25 January 2016 Interval 3:2 to Interval 17:2 |
| --- | --- |
| DA Number | 15824 and 15826 |
| Details | Severe lightning activity near the MRT-NOR81 line resulted in the line being taken out of service at 03:50. |
| System Management action | NAMKKN_MERR_SG1 and INVESTEC_COLLGAR_WF1 were constrained to 0MW to maintain Power System Security and Reliability. |

| Date and Interval/s | 3 February 2016 Interval 5:1 |
| --- | --- |
| DA Number | 15856 |
| Details | NEWGEN_KWINANA_CCG1 tripped at 5:02 with a loss of generation of approximately 200MW and frequency deviation to approximately 49.64Hz. |
| System Management action | No Out of Merit Generation was required and System Management continued to Dispatch as per the latest Balancing Merit Order |

| Date and Interval/s | 5 February 2016 Interval 11:2 |
| --- | --- |
| DA Number | 15861 |
| Details | NEWGEN_KWINANA_CCG1 tripped at 11:51am resulting in a loss of 320MW. Frequency dropped to approximately 49.49 Hz. |
| System Management action | No Out of Merit Generation was required and System Management continued to Dispatch as per the latest Balancing Merit Order |

| Date and Interval/s | 8 February 2016 Interval 11:1 to 9 February 2016 Interval 6:1 |
| --- | --- |
| DA Number | 15867 |
| Details | The LWT-SHO/KEM91 and KW-KEM/OLY91 330kV lines tripped at approximately 11:04am due to a major bushfire in the South West Region. The two 330kV lines were returned to service at approximately 11:11am however the bushfire still threatened the transmission lines. |
| System Management action | No Out of Merit was required as a consequence of the bushfire in the South West, however at 16:54 due to higher than forecasted load some Out of Merit Dispatch was required to meet the demand (refer DA15872). The following facilities were dispatched Out of Merit. AlCOA WGP, ALINTA_PNJ_U1, ALINTA_PNJ_U2, ALINTA_WGP_GT, BW1_BLUEWATERS_G2, BW2_BLUEWATERS_G1, MUJA_G3, NEWGEN_NEERABUP_GT1 and TIWEST_COG1. |

## Emergency Operating State

During the reporting period, no Emergency Operating State was issued.

Figure 4 below provides historical data for High Risk and Emergency Operating States that have occurred since 1 Oct 2015.

Figure 4: Number of High Risk and Emergency Operating States

## Shortfalls in Ancillary Services

There were no instances of shortfalls in Ancillary Services for the period.

## Involuntary curtailment of load

There were no instances of involuntary curtailment of load.

# LFAS Facilities out of Merit Order

During the current reporting period there were no incidents where System Management was required to use LFAS Facilities outside of the LFAS Merit Order to operate the SWIS in a reliable and safe manner as per MR7B.3.8.

Note clause 7.6.1D of the Market Rules provides for Out of Merit Dispatch to avoid a High Risk Operating State or an Emergency Operating State or if the SWIS is in a High Risk Operating State or an Emergency Operating State.